Real-Time Neural Network Inference

on STM32F407 under FreeRTOS

Project Technical Report

Batu Sengezer

June 2026

github.com/batu-sengezer/stm32f407-realtime-ai

# 1. Project Overview

This project demonstrates the deployment of a quantized neural network classifier on a resource-constrained microcontroller, running as a real-time task under FreeRTOS with hardware cycle-accurate timing measurement. The objective is to prove that a trained machine learning model can operate deterministically within the timing constraints of an embedded real-time system — a requirement in safety-critical applications including avionics, automotive, and industrial control.

The implementation covers the full pipeline from model training on a host machine, through post-training quantization, to on-chip deployment with verified timing behaviour. Training is fully reproducible via fixed random seeds. Inference is measured against real UCI HAR test samples covering all 6 activity classes. All timing measurements are obtained using the ARM Cortex-M4 Data Watchpoint and Trace (DWT) hardware counter, providing sub-microsecond timing resolution.

## 1.1 Hardware Platform

| Component | Detail |
| --- | --- |
| MCU | STM32F407VGTx — ARM Cortex-M4F @ 168 MHz |
| RAM | 192 KB SRAM |
| Flash | 1 MB |
| FPU | Yes (single-precision hardware floating point) |
| DSP | Yes (SIMD multiply-accumulate instructions) |
| Board | STM32F407G-DISC1 Discovery Kit |
| Debug interface | ST-LINK/V2-1 (on-board, SWD) |
| Host machine | MacBook Pro M5 Pro (Apple Silicon, macOS) |

## 1.2 Development Environment

| Tool | Version | Purpose |
| --- | --- | --- |
| STM32CubeIDE | 2.1.1 | IDE, compiler (ARM GCC), flasher, debugger |
| STM32CubeMX | 6.17.0 | Hardware configuration and code generation |
| X-CUBE-AI | 10.2.0 | Neural network C code generation and analysis |
| ST Edge AI Core | 2.2.0 | Model analysis, validation, and optimization |
| FreeRTOS | 10.3.1 | Real-time operating system (CMSIS-RTOS v2) |
| Python | 3.12 | Model training, quantization, test sample generation |
| TensorFlow | 2.21.0 | Model training and TFLite conversion |

# 2. Dataset

The UCI Human Activity Recognition (HAR) dataset was used. It consists of recordings from 30 subjects performing activities of daily living while carrying a waist-mounted smartphone with embedded inertial sensors (accelerometer and gyroscope at 50 Hz).

| Property | Value |
| --- | --- |
| Source | UCI Machine Learning Repository |
| Subjects | 30 |
| Sensor modalities | 3-axis accelerometer + 3-axis gyroscope |
| Sampling rate | 50 Hz |
| Classes | 6 (Walking, Walking Upstairs, Walking Downstairs, Sitting, Standing, Laying) |
| Features per sample | 561 pre-computed time and frequency domain features |
| Training samples | 7,352 |
| Test samples | 2,947 |
| Feature range | Normalized to [-1, 1] |

The dataset provides pre-computed feature vectors rather than raw sensor signals. Each 561-element feature vector summarises a 2.56-second sliding window of sensor data with 50% overlap. This makes it directly suitable for classification without additional signal processing on the MCU — a deliberate choice that keeps the inference input size fixed and predictable, which is important for static memory allocation on the microcontroller.

# 3. Model Architecture and Training

## 3.1 Architecture

A fully-connected feed-forward network was designed with the microcontroller's memory constraints as the primary design constraint. The architecture uses only Dense layers with ReLU activations, which are fully supported by the X-CUBE-AI INT8 quantization pipeline without any compatibility issues.

| Layer | Input | Output | Parameters |
| --- | --- | --- | --- |
| Input | — | (561,) | 0 |
| Dense + ReLU | (561,) | (64,) | 35,968 |
| Dense + ReLU | (64,) | (32,) | 2,080 |
| Dense + Softmax | (32,) | (6,) | 198 |
| Total | — | — | 38,246 |

Training configuration: Adam optimizer, sparse categorical cross-entropy loss function, 15 epochs, batch size 32. Random seeds fixed at 0 (random, numpy, tensorflow, PYTHONHASHSEED) for full reproducibility across runs. Validation was performed on the held-out test split throughout training.

## 3.2 Training Results

| Metric | Value |
| --- | --- |
| Random seed | 0 (fixed for reproducibility) |
| Final training accuracy | ~98% |
| Final validation (test) accuracy | 95.15% |
| Loss function | Sparse categorical cross-entropy |
| Optimizer | Adam (default learning rate 0.001) |
| Training time (M5 Pro) | < 15 seconds for 15 epochs |

# 4. Post-Training Quantization

Full integer quantization (INT8) was applied using TensorFlow Lite post-training quantization. This converts all weights and activations from 32-bit floating point to 8-bit integers, reducing model size by approximately 4x and enabling efficient execution on the Cortex-M4's integer arithmetic units.

## 4.1 Quantization Configuration

- Optimization: DEFAULT (post-training full integer quantization)

- Representative dataset: 100 calibration samples from the training set

- Input type: INT8 (symmetric, per-tensor quantization)

- Output type: INT8

- Weight quantization: symmetric per-channel INT8

- Supported operations: TFLITE_BUILTINS_INT8 only

## 4.2 Quantization Results

| Metric | FP32 Model | INT8 Model | Change |
| --- | --- | --- | --- |
| Accuracy | 95.15% | 94.98% | 0.17% degradation |
| Model file size | 479.0 KB | 42.5 KB | 11.3x smaller |
| Weight precision | float32 | int8 | 4x reduction per weight |
| Inference input | float32 | int8 | — |
| Inference output | float32 | int8 | — |

A 0.17% accuracy degradation is negligible for embedded deployment — well within the accepted threshold of 1% for post-training quantization. The representative dataset calibration successfully captured the activation ranges across all layers. The 11.3x size reduction (versus the theoretical 4x) is because the H5 file format carries additional overhead beyond raw weights.

FP32 accuracy is now computed dynamically in quantize.py rather than hardcoded, ensuring consistency when rerunning with different seeds or architectures.

# 5. On-Chip Resource Analysis

The INT8 TFLite model was imported into X-CUBE-AI 10.2.0 and analyzed by ST Edge AI Core (stedgeai), targeting the stm32f4 architecture. This tool converts the model to optimized C code and reports precise memory requirements before any code is written to the chip.

## 5.1 Memory Footprint

| Resource | Used | Available | Utilization |
| --- | --- | --- | --- |
| Flash (total) | 51.27 KB | 1,024 KB | 5.0% |
| RAM (total) | 4.24 KB | 192 KB | 2.2% |
| — Weights (flash) | 37.65 KB | — | — |
| — Runtime library (flash) | 13.62 KB | — | — |
| — Activations (RAM) | 2.34 KB | — | — |
| — Runtime library (RAM) | 1.90 KB | — | — |
| MACC operations | 38,336 | — | — |

The model occupies 5% of flash and 2.2% of RAM. This leaves substantial headroom for the FreeRTOS kernel, task stacks, and application code. Input and output buffers are allocated within the activations buffer (--allocate-inputs, --allocate-outputs flags), meaning no additional RAM is required for I/O beyond what is already counted in the 4.24 KB total.

## 5.2 Computational Complexity

| Layer | Type | MACC operations | % of total |
| --- | --- | --- | --- |
| Dense 561→64 | Fully connected (INT8) | 35,968 | 93.8% |
| Dense 64→32 | Fully connected (INT8) | 2,080 | 5.4% |
| Dense 32→6 | Fully connected (INT8) | 198 | 0.5% |
| Softmax | Nonlinearity (INT8) | 90 | 0.2% |
| Total | — | 38,336 | 100% |

The first dense layer (561→64) dominates at 93.8% of total operations. All operations use the smul_s8_s8 kernel (signed 8-bit multiply-accumulate), which maps directly to the Cortex-M4's DSP SIMD instructions. No floating-point operations are performed during inference.

## 5.3 Desktop Validation

Before on-target deployment, the generated C model was validated against the original TFLite model on the host machine using ST Edge AI Core's inspector:

| Validation Metric | Value |
| --- | --- |
| Cross-accuracy (TFLite vs C model) | 100.00% |
| Cosine similarity | 0.999998 |
| RMSE | 0.000713 |
| MAE | 0.000130 |
| Nash-Sutcliffe efficiency | 0.999993 |

The 100% cross-accuracy confirms that the C code generation introduced no additional classification errors beyond those already present in the quantized TFLite model.

# 6. FreeRTOS Real-Time Architecture

FreeRTOS 10.3.1 was integrated via the CMSIS-RTOS v2 API (CMSIS_V2 interface in STM32CubeMX). Three tasks with distinct priorities were created to simulate a realistic embedded system with competing workloads — the key condition required for meaningful real-time characterisation.

## 6.1 Task Configuration

| Task | Entry Function | Priority | Stack | Role |
| --- | --- | --- | --- | --- |
| InferenceTask | StartDefaultTask | osPriorityNormal | 128 words | Run neural network inference |
| SensorTask | StartSensorTask | osPriorityAboveNormal | 128 words | Load real test sample, trigger inference |
| LogTask | StartLogTask | osPriorityBelowNormal | 128 words | Accumulate timing statistics |

## 6.2 Inter-Task Communication

| Object | Type | Configuration | Purpose |
| --- | --- | --- | --- |
| dataReadySem | Binary semaphore | Dynamic allocation | SensorTask signals InferenceTask |
| logQueue | Message queue | 16 items x 4 bytes (uint32_t) | InferenceTask sends cycle counts to LogTask |

## 6.3 Task Behaviour

SensorTask (AboveNormal priority)

Copies the next pre-quantized UCI HAR test sample into the model input buffer via AI_FillInput(), then releases dataReadySem. Cycles through 12 real test samples — 2 per activity class — covering all 6 classes (Walking, Walking Upstairs, Walking Downstairs, Sitting, Standing, Laying). Waits 100 ms between samples via osDelay(100). Because SensorTask runs at AboveNormal priority, it preempts InferenceTask on every trigger, creating realistic scheduler interference during timing measurement.

InferenceTask (Normal priority)

Blocks indefinitely on dataReadySem (osWaitForever). On each semaphore release:

- Reads DWT->CYCCNT (cycle counter start)

- Calls MX_X_CUBE_AI_Process() — one complete inference pass on real input data

- Reads DWT->CYCCNT (cycle counter stop)

- Computes elapsed cycles = stop - start

- Posts cycle count to logQueue via osMessageQueuePut

LogTask (BelowNormal priority)

Receives cycle counts from logQueue via osMessageQueueGet (blocking). Accumulates statistics across 1,000 iterations using running totals:

- Running minimum (uint32_t)

- Running maximum (uint32_t)

- Running sum (uint64_t, to prevent overflow at 1000 iterations)

- Running sum-of-squares (uint64_t, for numerically stable variance computation)

On completion of 1,000 iterations, converts all values to microseconds (dividing by 168.0) and stores results in four volatile float globals: result_mean_us, result_min_us, result_max_us, result_sigma_us. Sets stats_ready = 1 to signal completion. Also toggles PD12 (green LED) on each inference as a visual heartbeat, and PD13 (orange LED) every 100 inferences.

## 6.4 Real Sensor Input

12 UCI HAR test samples are stored as a const int8_t array in flash (Core/Inc/test_samples.h), generated by generate_test_samples.py. Samples are pre-quantized using the model's input scale (0.007843) and zero-point (-1), matching the INT8 format the network expects.

| Class | Samples stored |
| --- | --- |
| Walking | 2 |
| Walking Upstairs | 2 |
| Walking Downstairs | 2 |
| Sitting | 2 |
| Standing | 2 |
| Laying | 2 |

SensorTask cycles through all 12 samples repeatedly, ensuring every activity class is represented across the 1,000 measurement runs. The AI_FillInput() function (defined in app_x-cube-ai.c) writes the sample directly into the ai_input buffer, bypassing the data_ins pointer layer to correctly target the activations-allocated input buffer.

# 7. Timing Measurement and Results

## 7.1 DWT Cycle Counter

The ARM Cortex-M4 Data Watchpoint and Trace (DWT) unit provides a 32-bit free-running CPU cycle counter. It is the gold standard for precise on-chip timing in safety-critical embedded systems. Enabled once at startup with three register writes:

CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;  // enable trace unit

DWT->CYCCNT = 0;                                  // reset counter

DWT->CTRL   |= DWT_CTRL_CYCCNTENA_Msk;           // start counting

At 168 MHz, each counter increment corresponds to 5.95 nanoseconds. Conversion to microseconds: time_us = cycles / 168.0. By comparison, HAL_GetTick() increments once per millisecond — entirely unsuitable for measuring inference latency in the sub-millisecond range.

## 7.2 Measurement Conditions

- 1,000 consecutive inference runs

- Real UCI HAR test samples cycling through all 6 activity classes

- SensorTask (AboveNormal priority) active and preempting throughout

- LogTask (BelowNormal priority) active and consuming queue items throughout

- All three tasks running concurrently under FreeRTOS preemptive scheduler

- Results extracted via CubeIDE debugger Expressions panel at breakpoint on stats_ready = 1

- Total measurement duration: approximately 100 seconds (1,000 x 100 ms sensor interval)

## 7.3 Results

| Metric | Value |
| --- | --- |
| Mean inference latency | 786.73 µs |
| Minimum inference latency | 786.58 µs |
| Maximum inference latency (WCET) | 786.89 µs |
| Jitter (standard deviation σ) | 2.24 µs |
| Range (max - min) | 0.31 µs |
| Total measurement runs | 1,000 |
| Input data | Real UCI HAR samples, all 6 activity classes |
| Scheduler load during measurement | SensorTask + LogTask active |
| Measurement method | DWT hardware cycle counter at 168 MHz |

## 7.4 Analysis and Interpretation

With real input data cycling through all 6 activity classes, the timing shows genuine data-dependent variation. The 0.31 µs range and 2.24 µs standard deviation reflect different computation paths through the INT8 multiply-accumulate operations depending on input values — different activity class feature vectors cause slightly different arithmetic in the smul_s8_s8 kernels.

Why jitter is small but non-zero

- No data cache: The Cortex-M4 has no L1 data cache. All RAM accesses hit SRAM directly in a fixed number of clock cycles — there are no cache miss effects. The variation comes purely from input-dependent arithmetic, not memory access patterns.

- Static memory allocation: All activation buffers are statically allocated at compile time. There is no dynamic memory allocation during inference, eliminating heap-related timing variability.

- Interrupt isolation: InferenceTask holds the CPU for the full duration of MX_X_CUBE_AI_Process(). SensorTask only preempts between inference runs, not during. The DWT measurement window is never interrupted by a context switch.

- Data-dependent arithmetic: Different input values cause slightly different execution paths in the INT8 SIMD kernels, producing structured variation across activity classes rather than random noise.

WCET significance

The worst-case execution time of 786.89 µs establishes a hard upper bound on inference latency under real input conditions. The SensorTask period of 100 ms gives a CPU utilisation of 786.89 / 100,000 = 0.79%, leaving 99.2% of CPU time available for other tasks.

Jitter of 2.24 µs represents 0.28% of mean latency — well within acceptable bounds for any real-time scheduling framework. Under Rate Monotonic Analysis, the task remains schedulable with substantial margin.

# 8. Project Outputs

## 8.1 Complete Numbers Table

| Category | Metric | Value |
| --- | --- | --- |
| Model | Dataset | UCI HAR (6 classes, 561 features) |
| Model | Architecture | FC 561→64→32→6, ReLU, Softmax |
| Model | Random seed | 0 (fixed) |
| Model | FP32 accuracy | 95.15% |
| Model | INT8 accuracy | 94.98% |
| Model | Accuracy drop | 0.17% |
| Model | FP32 size | 479.0 KB |
| Model | INT8 size | 42.5 KB |
| Model | Size reduction | 11.3x |
| On-chip | Flash used | 51.27 KB / 1,024 KB (5.0%) |
| On-chip | RAM used | 4.24 KB / 192 KB (2.2%) |
| On-chip | MACC operations | 38,336 |
| On-chip | Desktop validation accuracy | 100.00% (vs TFLite reference) |
| Timing | Mean latency | 786.73 µs |
| Timing | Min latency | 786.58 µs |
| Timing | WCET (max latency) | 786.89 µs |
| Timing | Jitter (σ) | 2.24 µs |
| Timing | Measurement runs | 1,000 — real UCI HAR samples, all 6 classes |
| Timing | CPU utilisation | 0.79% at 100 ms period |

## 8.2 Software Artefacts

| File | Description |
| --- | --- |
| train.py | Model training — UCI HAR, FC network, seed=0, saves model_fp32.h5 |
| quantize.py | INT8 quantization — converts FP32 to TFLite, evaluates accuracy dynamically, reports size |
| generate_test_samples.py | Generates test_samples.h — 12 UCI HAR test samples pre-quantized to INT8 as C array |
| rt_inf_latency.py | Latency visualization — mean and WCET markers, observed range band |
| accuracy_size_chart.py | FP32 vs INT8 comparison chart — accuracy and model size side-by-side |
| model_int8.tflite | Quantized INT8 TFLite model (42.5 KB) |
| latency_histogram.png | Inference timing visualization |
| accuracy_size_chart.png | Quantization impact chart |
| f407_ai_rt/Core/Src/main.c | DWT setup, InferenceTask, SensorTask with AI_FillInput() |
| f407_ai_rt/Core/Src/freertos.c | LogTask with statistics accumulation |
| f407_ai_rt/Core/Inc/test_samples.h | 12 real UCI HAR test samples as INT8 C array |
| f407_ai_rt/X-CUBE-AI/App/app_x-cube-ai.c | X-CUBE-AI runtime + AI_FillInput() implementation |
| f407_ai_rt/f407_ai_rt.ioc | CubeMX configuration — pins, clocks, FreeRTOS, X-CUBE-AI |

# 9. Known Limitations and Planned Extensions

## 9.1 Current Limitations

UART output not implemented

Timing results are currently extracted via the CubeIDE debugger at a breakpoint. The intended production approach is to route printf to USART2 (retargeted via a custom _write() function calling HAL_UART_Transmit), stream per-inference cycle counts to the host over serial, and generate the latency histogram from real on-device data. This would also enable longer measurement runs and real-time monitoring.

## 9.2 Planned Extensions

- USART2 integration with retargeted printf for real-time per-inference cycle count streaming

- Image classifier variant (Option A): 32x32 grayscale input, Conv2D architecture

- CMSIS-NN optimized kernels: rebuild with X-CUBE-AI CMSIS-NN target and measure speedup

- FPGA implementation via HLS: port the INT8 inference pipeline to FPGA fabric for custom datapath design, enabling hardware-software co-design exploration and spatial parallelism across the 38,336 MACC operations.

# 10. Reproducing the Results

## 10.1 Prerequisites

- STM32F407G-DISC1 Discovery board

- Mini-USB to USB-A cable (data-capable, not charge-only)

- STM32CubeIDE 2.1.1 (Mac ARM build)

- STM32CubeMX 6.17.0

- Python 3.12 with TensorFlow 2.21.0, numpy, matplotlib

- UCI HAR Dataset downloaded from UCI ML Repository

## 10.2 Steps

Step 1: Train, quantize, and generate test samples

python train.py                   # seed=0, produces model_fp32.h5, FP32 acc=95.15%

python quantize.py                # produces model_int8.tflite, INT8 acc=94.98%

python generate_test_samples.py   # produces test_samples.h

Step 2: Copy test samples header

cp test_samples.h path/to/f407_ai_rt/Core/Inc/

Step 3: Deploy to hardware

- Open f407_ai_rt in STM32CubeIDE 2.1.1

- Connect board via Mini-USB ST-LINK port

- Click Build (hammer icon) — should complete with 0 errors

- Click Run (green arrow) — flashes board and starts execution

- Green LED (PD12) toggles on each inference as visual confirmation

Step 4: Measure timing

- In CubeIDE, switch to Debug configuration

- Set a breakpoint on stats_ready = 1 in main.c

- Click Debug — board is flashed and execution begins

- After approximately 100 seconds (1000 x 100 ms), breakpoint fires

- In the Expressions panel, read: result_mean_us, result_min_us, result_max_us, result_sigma_us

github.com/batu-sengezer/stm32f407-realtime-ai